Practical Brainstorm marking guidance (so it’s consistent)

When you see a boundary line:

- If the signal looks stable immediately adjacent: leave it as boundary only.

- If you see a transient/amplitude shift/ringing: mark BAD spanning the unstable region.

- Prefer one continuous BAD segment rather than many tiny ones (easier QC and more stable union masks).

Below is a standard SOP add-on you can drop into your Methods/SOP document. It is written to be clear, auditable, and consistent with your pipeline decision: exclude only boundary + manually marked BAD, and do not censor recurring QRS.

SOP: EEG Time-Exclusion Marking (Boundary + Manual BAD) in Brainstorm

Purpose

To create a reproducible, audit-ready set of EEG time exclusions that removes (i) known recording discontinuities and (ii) visually identified artifact segments, while avoiding overly aggressive censoring of recurring physiological events.

These exclusions are later exported and merged into a per-run “union exclusion” file used for downstream EEG feature extraction and cross-modal alignment/masking.

Scope

Applies to all EEG runs imported into Brainstorm for this project (protocol: eegfmri_R01_ICRej70), using the *_desc-ICRej70_clean.set datasets.

Definitions

- boundary event: Discontinuity markers inherited from EEGLAB preprocessing (e.g., gradient artifact subtraction boundaries or recording breaks). These are treated as excluded intervals.

- BAD segments: Manually marked time intervals in Brainstorm that reflect visually identified artifacts not already captured by boundary.

- QRS markers: Cardiac event markers (if present). These are not used for censoring unless they occur within an excluded interval.

Required event groups (naming is case-sensitive)

- boundary (lowercase)

- BAD (uppercase)

Inputs

- Brainstorm protocol database: eegfmri_R01_ICRej70

- Raw EEG runs imported from: *_desc-ICRej70_clean.set

- Existing event groups from the dataset (may include QRS, Sync, triggers)

Outputs

Per run, after export:

- *_bst_exclusions.tsv containing at minimum:

  - boundary start/end times (seconds)

  - BAD start/end times (seconds)

After merge:

- *_excl_union.tsv (union and merged intervals from boundary ∪ BAD)

Responsibilities and consistency requirements

- The same operator should apply marking criteria across runs where possible.

- Event group names must be consistent:

  - manual segments must be recorded under BAD (uppercase).

- Exclusions must be limited to:

  - boundary and BAD only.

  - QRS events must not be converted into BAD bands.

Procedure

Step 1 — Load and display run

- Open Brainstorm and select protocol eegfmri_R01_ICRej70.

- Navigate to the subject and select the relevant raw EEG run (imported from *_desc-ICRej70_clean.set).

- Open the EEG viewer for the raw run.

- Confirm that event groups include:

  - boundary (present from EEGLAB)

  - BAD (to be created/used for manual marking)

Expected output: EEG viewer displays the time series with visible event markers, including boundary.

Step 2 — Treat all boundary events as excluded

- Do not delete or rename the boundary event group.

- Use boundary markers as the primary anchors for identifying discontinuities.

- For each boundary marker:

  - Inspect a short window before and after the boundary to assess signal stability.

Expected output: boundary events remain intact and will be exported as exclusion intervals.

Step 3 — Identify segments requiring manual exclusion (BAD)

Manual BAD segments are used to exclude time periods that are clearly unusable but not automatically marked as boundary.

Mark as BAD if you observe any of the following:

- Severe attenuation or near-flatlining of multiple channels (signal dropouts)

- Abrupt amplitude jumps or discontinuous baseline shifts not already fully captured by boundary

- Prolonged movement artifacts, electrode pops, or unstable segments (large non-physiological deflections)

- Post-boundary transients (ringing/instability) extending beyond the instantaneous boundary marking

Do not mark as BAD solely because:

- QRS-related transients occur (cardiac markers). These are retained unless embedded within an otherwise bad segment.

- Regular oscillatory activity varies naturally.

Expected output: A clear set of manual BAD segments that capture only genuinely unusable intervals.

Step 4 — Mark BAD segments (Brainstorm GUI)

- In the EEG viewer, select the time interval to exclude (click-drag on the time axis).

- Add the selection as a new event/segment under the event group BAD (uppercase).

- Prefer fewer, longer BAD intervals rather than many very short segments when describing the same artifact episode.

Guidance: If a boundary is immediately followed by unstable signal, mark BAD to span the unstable portion (do not attempt to encode this by adding QRS-based bands).

Expected output: Event group BAD contains one or more intervals with start/end times in seconds.

Step 5 — Standardize treatment around boundary

For each boundary marker:

- Inspect whether instability extends beyond the boundary marker.

- If yes:

  - Create a BAD segment covering the unstable portion adjacent to the boundary.

- If no:

  - Do not add BAD around the boundary.

Expected output: Only boundaries that truly show adjacent instability receive additional BAD padding.

Step 6 — Save and verify per run

- Confirm event group names:

  - boundary exists and contains expected intervals

  - BAD exists and contains your manual segments

- Spot-check that:

  - BAD intervals align with clearly problematic data portions

  - BAD intervals are not accidentally placed around every QRS marker

Expected output: Run is ready for automated export; manual markings are complete and consistent.

Quality Control (QC) checks (per run)

Perform these quick checks before export:

- Event group check: boundary and BAD exist and contain intervals.

- Coverage check: BAD segments do not dominate the run (e.g., should not remove a large fraction unless the run is genuinely poor).

- Boundary adjacency check: any obvious post-boundary instability is captured by BAD.

Expected output: Clean, conservative exclusions that focus on discontinuities and obvious artifacts.

Export and downstream use (summary)

After completing manual marking across runs:

- Run the batch export script to write *_bst_exclusions.tsv (boundary + BAD).

- Merge exclusions into *_excl_union.tsv (union + interval merging).

- Use the union exclusions later to compute TR masks and to apply lag-safe dilation (if/when EEG lags are used).

Expected output: A complete exclusion set per run suitable for standardized downstream masking.

One-page checklist: EEG exclusions in Brainstorm (Boundary + Manual BAD)

Protocol: eegfmri_R01_ICRej70
Input EEG: *_desc-ICRej70_clean.set
Goal: Create conservative, reproducible exclusions using only boundary + manual BAD (do not censor QRS).

A) Pre-flight (per run)

- Open subject → open raw EEG run in Brainstorm viewer

- Confirm event groups include:

  - boundary (lowercase)

  - BAD (uppercase; create if missing)

- Confirm boundary markers visually align with discontinuities

Rule: Do not rename event groups (case-sensitive).

B) Boundary handling (per run)

For each boundary marker:

- Inspect a brief window before and after the boundary

- If signal is stable immediately adjacent → leave as boundary-only

- If there is transient/instability beyond the boundary marker:

  - Create a BAD segment covering the unstable portion

Typical signs needing BAD near boundary: ringing, amplitude jump, baseline shift, post-boundary disturbance.

C) Manual BAD marking (per run)

Mark a segment as BAD if you see:

- Severe attenuation / dropout / near-flatlining across many channels

- Abrupt amplitude jump or discontinuity not fully captured by boundary

- Movement artifact / electrode pop / unstable segment (non-physiological deflections)

- Prolonged “ugly” segment that would contaminate envelopes/band power

Do not mark as BAD solely because:

- QRS markers appear (recurring cardiac events)

- Normal variability in oscillations

Style guidance:

- Prefer fewer, longer BAD segments over many tiny ones for the same artifact episode

- Ensure BAD segments capture the entire unstable interval (not just its peak)

D) QRS handling (explicit)

- Do not create narrow BAD windows around QRS markers

- QRS events are informational only unless they fall inside an already excluded segment

E) Save + verify (per run)

- Confirm BAD segments are in the BAD (uppercase) event group

- Confirm boundary is unchanged and still present

- Quick sanity check: excluded time does not dominate the run unless truly necessary

- Visually confirm no “accidental” BAD marking around recurring QRS

F) Ready for export (batch stage after all runs)

- Run batch export → produces *_bst_exclusions.tsv (must include boundary and BAD)

- Run merge union → produces *_excl_union.tsv

- Run QC summary → review excluded fraction and any flags

Documentation line to include in SOP/Methods

“We excluded time periods corresponding to acquisition discontinuities (boundary) and additional visually identified artifacts (BAD) marked in Brainstorm. Recurring cardiac events (QRS) were not used for censoring unless they occurred within excluded segments.”