fMRI inputs and space

Resting-state BOLD fMRI data were preprocessed with fMRIPrep (see derivatives for full preprocessing provenance). For subsequent parcel time-series extraction, we used the volumetric BOLD series in MNI152NLin2009cAsym space (*_space-MNI152NLin2009cAsym_desc-*_bold.nii.gz), together with the matching brain mask (*_space-MNI152NLin2009cAsym_desc-brain_mask.nii.gz) and the run-level confound files (*_desc-confounds_timeseries.tsv and .json).

Atlas definition and resampling to each run’s BOLD grid

We used the Schaefer2018 atlas (200 parcels; 7 networks) in MNI space. The atlas was resampled to each run’s BOLD reference grid using nearest-neighbor interpolation to preserve integer parcel labels. To ensure voxelwise correspondence between the atlas and BOLD data, all parcel assignments were restricted to voxels inside the fMRIPrep brain mask. The resulting “atlas-on-grid” image was saved for debugging and reproducibility (*_atlas_on_grid.nii.gz).

Voxelwise nuisance regression (run-wise)

For each run, we performed nuisance regression in voxel space (restricted to brain-mask voxels). Let Y∈RT×Vdenote the masked BOLD voxel time series, where Tis the number of TRs and Vthe number of in-mask voxels. We fit an ordinary least squares (OLS) model:

Y=Xβ+ε

and retained residuals:

Yres=Y-Xβ.

Continuous confounds (z-scored)

Continuous nuisance regressors were assembled from fMRIPrep confounds and z-scored prior to regression for numerical conditioning. The continuous confound set included:

- Motion 24-parameter model: 6 rigid-body motion parameters (translations/rotations), their temporal derivatives, squares, and squared derivatives.

- Tissue signals: white matter (WM) and cerebrospinal fluid (CSF), including derivative and quadratic expansions where available.

- Low-frequency drift terms: cosine basis terms (cosine*) when present.

- aCompCor: up to N=10aCompCor components; when available, we preferentially used components marked as Retained=True in the fMRIPrep confounds JSON.

- Non-steady-state regressors: non_steady_state_outlier* columns when present.

Spike regressors for motion/outlier volumes

To reduce the impact of transient artifacts without discarding TRs, we modeled motion-related outlier volumes using spike (one-hot) regressors:

- FD spikes: volumes with framewise displacement (FD) >0.5 mm.

- Catastrophic FD expansion: for very large motion events (FD >2.0 mm), we additionally flagged the subsequent volumes (t, t+1, t+2) to capture potential post-motion contamination.

- fMRIPrep motion outliers: we included motion_outlier* regressors only when the corresponding outlier occurred at FD ≤0.5 mm. This specifically targets intensity transients that are not well captured by FD thresholding.

An intercept term was included in all models. DVARS-based spike regressors were computed as diagnostics but were not included in the final nuisance model due to overly aggressive censoring observed in early QC.

Parcel time-series extraction via PC1

Parcel time series were computed from the nuisance-regressed residuals Yres. For each parcel k∈{1,…,200}, we extracted all voxel residuals belonging to that parcel, forming Yk∈RT×Vk. If Vk<10, we used the parcel mean as a fallback summary. Otherwise, we performed PCA using SVD on mean-centered (and optionally voxelwise z-scored) voxel residuals and retained the first principal component (PC1) as the parcel time series.

PC sign convention

Because PCA sign is arbitrary, we enforced a deterministic sign convention: PC1 was flipped (multiplied by -1) when necessary so that it had nonnegative correlation with the parcel mean time series. This stabilizes parcel time-series polarity across runs and reruns.

Outputs and provenance

For each run, we saved:

- parcel PC1 time series (*_parcel_pc1.npy, shape T×200)

- parcel labels/names and voxel counts (*_parcel_labels.npy, *_parcel_names.npy, *_parcel_nvox.npy)

- MATLAB export (*_parcel_PCs_final.mat)

- per-run QC figure overlaying PC traces and FD (*_qc_pc1_fd.png)

- dataset_index.csv capturing file provenance, nuisance-model settings, and QC metrics.

Quality control and reproducibility checks

QC included:

- FD summary metrics per run (mean, 95th percentile, max, and % of FD spikes).

- Motion leakage check: correlation between FD and each parcel PC1; summarized by median/p95/max absolute correlation.

- Artifact-structure QC (“parcel blowup fraction”): for each TR, we computed the fraction of parcels with ∣z∣>3; we flagged TRs with ≥30% parcels exceeding this threshold.

- Reproducibility/sign-convention QC: for each run, we recomputed PC1 for a random subset of parcels directly from voxel residuals and confirmed near-identical correlation with the saved parcel outputs.

Results

Run-level motion characteristics

Across 15 runs, FD-based QC identified two runs with elevated motion characteristics (high FD spike fraction and/or extreme FD max), while the remaining runs were within acceptable ranges for resting-state analyses. After implementing catastrophic FD expansion and filtered fMRIPrep motion-outlier regressors, FD spike fractions increased slightly only in runs containing large motion events, consistent with the intended “t..t+2” expansion.

Motion–signal leakage after nuisance regression

The FD→PC correlation QC indicated low residual coupling between FD and parcel PC1 time series across runs. Median absolute correlations were typically on the order of 10-2, and maximal absolute correlations remained small (worst-case <0.1), suggesting that the nuisance model effectively removed linear FD-associated variance from parcel signals.

Artifact-structure QC (“mass parcel blowups”)

Prior to incorporating filtered motion_outlier* regressors, one run exhibited a single time point where ~45% of parcels exceeded ∣z∣>3, despite FD being below the FD threshold at that TR, indicating an intensity transient not captured by FD. After adding fMRIPrep motion outlier regressors restricted to low-FD events, this mass-parcel outlier was eliminated: no runs contained any TR where ≥30% of parcels exceeded ∣z∣>3. The worst observed parcel blowup fraction decreased to ≤0.20 across runs, with no evidence of sustained instability.

Reproducibility/sign-convention verification

In reproducibility QC (qc_sign_summary.csv), all runs achieved 100% pass rate (25/25 tested parcels per run) with correlations of 1.0 between saved and recomputed parcel PC1 time series, confirming that atlas mapping, nuisance regression, PCA extraction, and sign conventions were implemented deterministically.